Face Detection

Multi-Factor Driver Authentication System  ·  Phase 1

# 1. Overview

Phase 1 implements the core computer-vision pipeline: face detection, driver registration, real-time face recognition, and automatic LED lighting control. It is software-only and runs on a standard PC or Raspberry Pi with an attached USB/CSI camera.

## 1.1 Project Context

Modern commercial vehicles require secure authentication to prevent unauthorized use. Phase 1 replaces key-based access with face recognition, using a multi-pose registration process (inspired by iPhone Face ID) to build a robust facial model that works under varying lighting conditions.

## 1.2 Key Features

- Strict Haar Cascade face detection (centre-zone only, high confidence)

- iPhone-style multi-pose registration — 5 poses × 2 lighting conditions × 20 samples = 200 samples

- LBPH (Local Binary Patterns Histogram) face recognizer

- Multiple-face alert — access blocked if more than one person is detected

- Debounced LED relay control — hysteresis prevents flickering

- CSV event logging with timestamps

Phase Scope Phase 1 covers face detection, registration, and authentication only. Fingerprint authentication and SD-card logging are later-phase features.

# 2. System Architecture

## 2.1 Module Overview

| Module | Responsibility |
| --- | --- |
| Face Detection | Detects faces in the centre zone using OpenCV Haar Cascades with strict parameters (minNeighbors=8, minSize=100×100). |
| Registration | Guides the driver through 5 poses × 2 lighting conditions, capturing 20 frames per sub-pass, and trains LBPH. |
| Authentication | Compares the live face ROI against the stored LBPH model; grants or denies based on a confidence threshold. |
| LED Controller | Measures central-ROI brightness and debounces LED relay changes with a confirmation timer. |

## 2.2 System Flow

Camera Input → Face Detection → [Registration path / Authentication path] → LED Control → CSV Log.

System flow diagram — the full authentication pipeline

# 3. Face Detection

## 3.1 Method

Phase 1 uses OpenCV's Haar Cascade classifier (haarcascade_frontalface_default.xml). Detection is restricted to the centre zone of each frame — inset from the edges — to prevent background objects from triggering false detections.

## 3.2 Detection Parameters

| Constant | Value | Description |
| --- | --- | --- |
| DETECT_SCALE | 1.1 | Image-pyramid scale factor (smaller = more detections, slower) |
| DETECT_MIN_NEIGHBORS | 8 | Min neighbour rectangles (higher = fewer false positives) |
| DETECT_MIN_SIZE | (100, 100) | Minimum face size in pixels |
| DETECT_ZONE_X | 80 px | Horizontal inset of the detection zone |
| DETECT_ZONE_Y | 60 px | Vertical inset of the detection zone |

## 3.3 Multiple Face Handling

If more than one face is detected during authentication, a full-width alert banner is shown and access is denied until only one face remains. All multi-face events are logged.

| Single face — normal detection | Multiple-face alert — access blocked |
| --- | --- |

## 3.4 Detection During Registration

During registration, detection is deliberately relaxed (minNeighbors=4, minSize=60×60) to accommodate angled head poses that would otherwise fail the strict authentication-mode detector.

# 4. Driver Registration

## 4.1 Overview

Registration follows an iPhone Face ID-style guided capture. The driver completes five head-pose steps; each is captured twice — once under normal lighting and once under simulated low light — ensuring the model is robust to varying conditions.

Sample Count Each sub-pass captures 20 samples (SAMPLES_PER_STEP). Total = 5 poses × 2 lighting × 20 samples = 200 training samples stored in driver1_face.npy.

## 4.2 Registration Steps

| 1 | Look straight at the camera Baseline frontal view — captured in normal and low light |
| --- | --- |
| 2 | Slowly turn head RIGHT Right-profile variation |
| 3 | Slowly turn head LEFT Left-profile variation |
| 4 | Tilt head UP Upward-gaze variation |
| 5 | Tilt head DOWN Downward-gaze variation |

## 4.3 Registration UI

| Step 1 — Look straight (normal light) | Step 1 — Look straight (low light) |
| --- | --- |

| Step 2 — Turn right | Step 3 — Turn left |
| --- | --- |

| Step 4 — Tilt up | Step 5 — Tilt down |
| --- | --- |

Registration complete — “Driver 1 Registered!”

# 5. Authentication

## 5.1 Recognizer

Phase 1 uses OpenCV's LBPH face recognizer. LBPH divides the face ROI into a grid of cells, computes texture histograms per cell, and compares them to the training set. It performs well under minor lighting and pose changes.

## 5.2 Match Threshold

Lower confidence means a better match. The threshold is configured as:

MATCH_THRESHOLD = 75 # granted if label == 1 and confidence < 75

## 5.3 Authentication States

| State | Condition | Visual Feedback |
| --- | --- | --- |
| Access Granted | label=1, conf < 75 | Green box, 'DRIVER 1 — MATCHED', score |
| Access Denied | label≠1 or conf ≥ 75 | Red box, 'NOT MATCHED', 'Access Denied' |
| No Driver | driver1_face.npy missing | Orange banner — 'Press R to set up Face ID' |
| Multiple Faces | len(faces) > 1 | Red full-width alert banner |
| No Face | len(faces) == 0 | Status bar cleared — waiting |

## 5.4 Authentication Screenshots

| Access granted — green box, confidence shown | Access denied — red box, NOT MATCHED |
| --- | --- |

# 6. LED Auto-Lighting

## 6.1 Brightness Measurement

Each frame, the system measures average pixel brightness in a central region:

def measure_brightness(frame): h, w = frame.shape[:2] cx, cy = w // 2, h // 2 roi = frame[cy-75:cy+75, cx-100:cx+100] return float(np.mean(cv2.cvtColor(roi, cv2.COLOR_BGR2GRAY)))

## 6.2 Thresholds

| Constant | Value | Description |
| --- | --- | --- |
| BRIGHTNESS_LOW | 60 | Below this, LED turns ON (low light) |
| BRIGHTNESS_OK | 80 | Above this, LED turns OFF (good light) |
| LED_CONFIRM_SECS | 5.0 | Stable-state seconds before the LED switches |

## 6.3 Hysteresis / Flicker Fix

The LED controller debounces state changes: the LED only switches if brightness stays past the threshold for the confirmation window, preventing rapid toggling from noise or momentary shadows.

| LED ON — low light detected | LED OFF — good light |
| --- | --- |

## 6.4 Hardware Note

Hardware Flag RELAY_LED_HARDWARE = False by default. In software-only mode the LED state is printed and logged; physical relay control arrives in Phase 3.

# 7. Event Logging

## 7.1 CSV Log Format

| Field | Example | Description |
| --- | --- | --- |
| Timestamp | 2025-05-18 14:32:07 | Date and time of the event |
| Event | FACE_MATCH | Event type identifier |
| Detail | conf=42.3 | Additional context |
| Result | GRANTED | Outcome of the event |

## 7.2 Logged Events

- SYSTEM_STARTED, DRIVER_1_REGISTERED, FACE_MATCH, FACE_NO_MATCH, MULTIPLE_FACES, LED_RELAY, SYSTEM_STOPPED

## 7.3 Log Screenshot

auth_log.csv — timestamped event log

# 8. Configuration Reference

| Constant | Value | Description |
| --- | --- | --- |
| CAMERA_INDEX | 0 | Camera device index |
| FRAME_WIDTH / HEIGHT | 640 × 480 | Capture resolution |
| DRIVER_FACE_FILE | driver1_face.npy | Saved LBPH model |
| LOG_FILE | auth_log.csv | Event log path |
| MATCH_THRESHOLD | 75 | Max LBPH confidence for a match |
| SAMPLES_PER_STEP | 20 | Samples per lighting sub-pass |
| BRIGHTNESS_LOW / OK | 60 / 80 | LED on/off brightness thresholds |
| RELAY_LED_HARDWARE | False | Enable GPIO relay output |

# 9. Controls, Limitations & Next

## 9.1 Controls & Running

R — start driver registration (re-registration overwrites the model). Q — quit cleanly, releasing the camera and saving the log.

pip install opencv-python opencv-contrib-python numpy python driver_auth_phase1.py

## 9.2 Known Limitations

- Haar detection can fail at extreme head angles (>45°), mitigated by relaxed registration parameters.

- LBPH accuracy drops with drastic lighting changes not covered during registration.

- Only one driver profile (Driver 1) is supported in Phase 1.

- LED control is software-only in Phase 1 (physical relay arrives in Phase 3).

## 9.3 What Comes Next — Phase 2

- Fingerprint sensor integration as a second authentication factor

- Dual-factor access decision combining face and fingerprint