Fingerprint Authentication

Multi-Factor Driver Authentication System  ·  Phase 2

# 1. Overview

Phase 2 adds fingerprint authentication as a second, independent identity factor. Where Phase 1 granted access on a face match alone, Phase 2 combines face recognition with a physical fingerprint scan from an R307S optical sensor, so the vehicle is only authorised when both factors agree.

## 1.1 Project Context

A face match alone can be spoofed with a photograph and offers no proof that the authorised driver is physically present. Phase 2 introduces a biometric “something you are” factor requiring live finger contact. The module runs in two modes — a software simulation for development on any PC, and a real-sensor mode driven by the R307S over UART on the Raspberry Pi — selected by a single configuration flag.

## 1.2 Key Features

- R307S optical fingerprint sensor over UART (/dev/ttyAMA0, 57600 baud) via the pyfingerprint library

- Two-scan guided enrollment with same-finger confirmation before the template is stored

- On-sensor template storage at slot #1, with a disk flag (fingerprint_enrolled.flag) so enrollment persists across restarts

- Dual-factor decision: access GRANTED only when face AND fingerprint both match

- Live UI feedback driven by four scan states — WAITING, SCANNING, LIFT, DONE

- Simulation mode (USE_REAL_SENSOR = False) for development without hardware

Phase Scope Phase 2 covers fingerprint enrollment, verification, and the dual-factor access decision. Relay-driven ignition control and SMS remote operation are documented in later phases.

# 2. Fingerprint Hardware

## 2.1 Sensor

Phase 2 uses the R307S optical fingerprint sensor, which performs image capture, feature extraction, template storage, and matching on-board. The Raspberry Pi communicates over a UART serial link; all matching happens inside the sensor, so no fingerprint images are stored on the Pi.

## 2.2 Wiring

The R307S exposes six wires. The blue TOUCH line signals finger presence and is read as an active-low GPIO input with an internal pull-up.

| Wire | Signal | Pi Pin | Notes |
| --- | --- | --- | --- |
| Red | VCC | Pin 4 (5V) | Sensor power |
| Black | GND | Pin 6 (GND) | Common ground with Pi |
| Yellow | TXD | Pin 10 (RX / GPIO15) | Sensor TX → Pi RX |
| Green | RXD | Pin 8 (TX / GPIO14) | Pi TX → Sensor RX |
| Blue | TOUCH | Pin 11 (GPIO17) | Active-low finger-presence |
| White | WAKEUP | — | Leave floating |

Serial Setup (one-time) Run sudo raspi-config → Interface Options → Serial Port. Set “Login shell over serial?” to No and “Serial hardware enabled?” to Yes, then reboot. On Pi 4 the sensor port is /dev/ttyAMA0.

## 2.3 Hardware Test Script

Code/Test/fingerprint_test.py validates the wiring before integration. It runs seven checks: sensor info (template count and capacity), the blue TOUCH wire, raw image capture over UART, a combined touch-then-capture flow, enrollment into slot 1, verification, and template deletion.

📷 R307S Hardware Test Output Add screenshot / photo here

# 3. Fingerprint Enrollment

## 3.1 Overview

Enrollment registers Driver 1's fingerprint once; it then persists for every future run. The driver scans the same finger twice — the second scan confirms the first — and the sensor combines them into a single template stored in its own memory.

Enroll Once, Verify Forever After a successful enrollment the template lives in the sensor at slot #1, and a fingerprint_enrolled.flag file is written to disk. Because the flag survives restarts, the system never asks the driver to re-enroll unless the flag is explicitly cleared.

## 3.2 Enrollment Steps

| 1 | Place finger on the sensor First scan — the sensor captures the image into CharBuffer 1 |
| --- | --- |
| 2 | Remove finger when prompted Brief pause before the confirmation scan |
| 3 | Place the SAME finger again Second scan into CharBuffer 2 — confirms the first capture |
| 4 | Template created and stored Sensor compares the two scans, builds the template, and stores it at slot #1 |

If the two scans do not match (compareCharacteristics returns 0), enrollment fails and must be retried. On success, the template is written with storeTemplate() and the enrollment flag is saved.

## 3.3 Enrollment UI

During enrollment the on-screen panel shows the live scan state and a step checklist that fills in as each stage completes. The flow auto-completes when the template is saved.

📷 Fingerprint Enrollment — In Progress / Complete Add screenshot / photo here

# 4. Verification & Dual-Factor

## 4.1 Verification

Verification captures a single live scan and calls searchTemplate(), which compares it against every template in the sensor's memory and returns the matching slot and a raw score. The module converts that raw score into a confidence percentage for display and logging.

result = f.searchTemplate() slot, score = result[0], result[1] if slot == -1: return False, confidence # no template matched (~20-45%) confidence = round(min(99.9, score / 2.0), 1) return True, confidence # MATCH at this slot

A verification attempt waits up to SENSOR_TIMEOUT (15 seconds) for a finger; if none is placed, it returns as a timeout (no match).

## 4.2 Scan States

| State | Meaning | On-screen Prompt |
| --- | --- | --- |
| WAITING | Sensor ready, no finger yet | “Place your finger on the sensor” |
| SCANNING | Finger detected, capturing image | “Hold still — Scanning” |
| LIFT | Capture done, finger can be removed | “Lift your finger” |
| DONE | Result ready (match / no match) | Shows MATCHED or NO MATCH with confidence |

## 4.3 Dual-Factor Decision

The face and fingerprint results are combined into a single access decision by logger.log_session(). In full authentication mode, both factors must succeed:

final = "GRANTED" if (face_matched and fp_matched) else "DENIED"

Face-Only Auto-Check A lightweight “auto-check” mode (face_only = True) skips the fingerprint and decides on the face alone. It is used for passive presence detection — not for granting ignition — and a failed auto-check raises the “UNAUTHORISED INDIVIDUAL DETECTED” alert. Full ignition-granting authentication always requires both factors.

# 5. Configuration & Usage

All fingerprint parameters are defined at the top of fingerprint_sim.py. The single most important flag is USE_REAL_SENSOR, which switches between the real R307S and the software simulation.

| Constant | Value | Description |
| --- | --- | --- |
| SERIAL_PORT | /dev/ttyAMA0 | Pi UART port for the R307S (COM port on Windows) |
| BAUD_RATE | 57600 | Sensor serial baud rate |
| SENSOR_TIMEOUT | 15 | Seconds to wait for a finger before timing out |
| ENROLLED_ID | 1 | Sensor template slot used for Driver 1 |
| USE_REAL_SENSOR | True | True = real R307S over UART; False = simulation |
| TOUCH_GPIO | 17 | BCM pin for the blue TOUCH wire (active-low) |

pip install pyfingerprint pyserial --break-system-packages python3 fingerprint_sim.py # built-in enroll / verify menu python3 Test/fingerprint_test.py # full 7-step hardware test

## 5.1 Known Limitations

- Only one driver fingerprint (slot #1) is enrolled in Phase 2; multi-driver enrollment is a later phase.

- Dry, wet, or dirty fingers reduce optical-sensor match scores and may require a re-scan.

- The R307S requires a common ground with the Pi and a stable 5V supply; a brownout causes UART init to fail.

- Confidence percentages are derived from the sensor's internal score and are indicative, not calibrated error rates.

## 5.2 What Comes Next — Phase 3

- 3-channel relay control via the timer latch, LED, and ignition channels

- Ignition enabled only on a successful dual-factor GRANTED decision